Sandskript / Sybil — Pipeline & Tile System

Status update — August 2026

Updated against the original planning notes below. Marked [DONE] where built and tested this session, [OPEN] where it's still genuinely outstanding, [NOTE] for reference/context that hasn't changed.

# Pipeline Structure

## Reference repo

- [NOTE] Separate copy of Ruth's repo as a starting point — home position, world object unchanged. No change.

## Receiving from Lin

- was: mqtt (new, separate module) — owns the actual subscribe loop.

- → Built differently than planned: ZeroMQ instead of MQTT (no broker needed for a one-machine setup, same code scales to multi-machine later by changing one address). main.py's autonomous mode owns the receive loop directly; a separate tile_announcer.py / tile_receiver.py pair handles the outbound direction (sending the next tile selection back to Lin).

- [OPEN]  Real message format/protocol with Lin's actual pipeline still needs confirming —

  - tested so far with synthetic data (301b_flat_path_to_tile.py) and a local test publisher, not her live output.

## main.py

- was: New, thin. Takes received data, calls in order: convert → process → visualize → send. No logic of its own beyond naming the order and passing the file path along.

- [DONE]  and thinner than planned — the actual step sequence now lives in one shared module (pipeline_utils.py), imported by main.py AND the two other entry points (300_watch_and_run.py, 300b_zmq_listener.py), so the sequence can't quietly drift between them. Supports two modes: single (one file, then exit) and autonomous (runs forever, receives new captures automatically).

## 301_convert_to_compas_json.py

- [DONE]  Reads the raw file, writes a converted COMPAS-frame JSON to data/compas. Tile ID now carried through untouched if present in the source file.

## 302_process_toolpath.py

- was: Reads the converted file, writes a robot-ready JSON.

- was: Aligns normals with surface center

- → Every frame's orientation is now rebuilt radially from a single shared center point (fixed_geometry.py), regardless of the tween blend.

- was: Moves path within robot reach — scaling? distances relative to an origin so the robot still reaches out? (BAUSTELLE) some sort of move/project/scale — think up how this could work

- → This is exactly what the tween/morph system does: every point blends between its original surface position and a projected position at a target radius from the shared center, then gets hard-clamped to a safety radius. No separate "reach-out" step needed — it's the same math.

- was: Scaling yes

- → Confirmed: Lin's pipeline outputs millimetres natively. No conversion applied (COORDINATE_SCALE = 1.0).

- was: Add safe frame here?

- → No — deliberately NOT added in 302. Safe frames are built in 304 instead, computed from the robot's live measured orientation. Adding them in both places caused a real double-offset bug earlier; fixed by keeping this in one place only.

- was: WO (0,0,0) add where?

- → fixed_geometry.py — one shared WORLD_ORIGIN. The robot's own base, the world object's origin, and the safety sphere's center are all the same literal point, referenced everywhere, not three separate values that could drift apart.

- [NOTE]  Output actually saves to data/processed, not data/compas as originally noted — data/compas is 301's output folder.

## view_utils.py

- [DONE]  Opens automatically on the robot-ready file and blocks until closed — as planned. Since then: fixed real-world scale and camera framing (was reading as “microscopic,” actually a camera-positioning issue, not a units mismatch — geometry was mm all along). Safety + target shells, spray surface mesh, and all colors/sizes pulled into adjustable constants.

## 304_send_to_robot.py

- was: Only runs after closing the viewer, reads the same robot-ready file, connects live, fixes orientation, executes.

[DONE]  , and substantially extended: a MODE switch (preview / execute / test_record) so nothing ever touches the robot by accident; an ORIENTATION_MODE switch (fixed-from-home vs per-point radial, switchable for testing both); six independent staged checkpoints (comms → position → home → safe frame → first point → full path); safe-frame retreat now calculated toward the shared center rather than a fixed world direction. After a real spray, automatically records the execution and runs tile selection + announcement (see below).

# Resolved Since Last Update

- [DONE]  Tile selector — random pick, checked against eligibility rules, retried on failure. Separate rate-limit (per rolling time window) and optional lifetime cap, independently for water and substrate.

- [DONE]  Record executed & write tile status — every real spray saves a full copy of its executed toolpath as its own JSON file (timestamp, tile, substance all in the filename). Status is always read live from these files, never a stored counter that could go stale.

- [DONE]  Testing paths + messaging setup — 301b_flat_path_to_tile.py for synthetic geometry; tile_receiver.py / tile_announcer.py and 300b_zmq_listener.py / 300b_zmq_publisher.py for testing the message layer without needing Lin's or the robot's real systems running.

- [DONE]  302 clearly defined — full docstrings, and the tween/safety-clamp logic rebuilt as proper classes (RadialShell, ToolpathMorph).

- [DONE]  304 + tile execution structure decided — no separate “305” file in the end. Tile recording, selection, and announcement all happen inside 304 right after a successful spray, calling out to three small dedicated modules (tile_status.py, tile_selector.py, tile_announcer.py).

- [DONE]  Loop that starts when new data arrives — main.py's autonomous mode, event-driven, no manual restart needed between captures.

- [DONE]  OOP homework — applied throughout: RadialShell/ToolpathMorph (302), Toolpath/RobotSession/ToolpathExecutor (304), SprayRecord/TileHistory/TileSelector/TileAnnouncer (the tile system).

# Still Open

- [OPEN]  Confirm real message format and origin/scaling against Lin's actual per-tile projected paths — flagged as possibly still off, since the tile being sent may carry an old origin. Not yet tested end-to-end with her live output.

- [OPEN]  First full circle real robot test, staged step by step.

- [NOTE]  “303” no longer exists as a separate file — renamed to 304 earlier in this work. Flagging so old notes referencing 303 aren't confusing later.

# Open Questions — Physical / Domain

- [OPEN]  Net prep: pre-coat with a clay mixture before spraying substrate.

- [OPEN]  Water distance / misting tests — spraying from further back may mist rather than jet; test next.

- [OPEN]  Watering mode: possibly a simple left-to-right sweep across the whole tile, rather than the point-by-point path used for substrate — would need its own toolpath-generation logic, not yet built.

- [OPEN]  Watering vs. spraying likely need different stored data per run, not just a label — current system already separates them by spray_type (water / substrate) with independent limits; whether that's sufficient or a deeper split is needed is still open.

- [OPEN]  Spray protocol questions: how many layers per path? Is it one path with four sprays? First spray then 30 minutes, then three rounds back to back? What are the actual rules behind “spray x3”? — needs a clear answer before it can be encoded anywhere.

# Meeting Notes

- [NOTE]  Tuesday — next check-in / QA meeting when?

Lin:

- Branched: set-up ZMQ

- Add: zmq_bridge.py

- Install: pyzmq>=25.0.0 (added to requirements.txt)

- Add: folder for testing “test_folder”