Separate copy of ruths repo as a starting point (see how she did things, learn, orientation)

Updated structure, stage by stage:

- mqtt (new, separate module) — owns the actual subscribe loop. When a message arrives from Lin receives from Lin. Joe  Monday help.

- main.py (new, thin) takes received data from mqtt. For each new file that arrives, it calls, in order: convert → process → visualize → send. No logic of its own beyond naming the order and passing the file path along. CALLS PRINT FUNCTION.

print("=== 301: convert ===")

subprocess.run(["python", "301_convert_to_compas_json.py", str(INPUT_PATH)], check=True)

subprocess.run([...]) takes a list — think of it as literally typing python 301_convert_to_compas_json.py <the path> into a terminal yourself, except Python does it for you. check=True means: if 301 crashes (a non-zero exit code), stop everything immediately with an error, rather than silently continuing to 302 with broken/missing data.

- 301_convert_to_compas_json.py — reads that raw file, writes a converted COMPAS-frame JSON to data/compas

- 302_process_toolpath.py — reads that converted file, writes a robot-ready JSON

  - Aligns normals with surface center

  - Moves path within robot reach

    - Scaling yes

    - Distances stay relative to an origin so the robot still does the reach out motion (BAUSTELLE  o.o) some sort of move project scale… think up how this could work!!! (weekend)

    - Add safe frame here?

    - WO (0,0,0) add where?

  - Safe json to data/compas.

- View_utils.py — opens automatically on that robot-ready file, and blocks — nothing continues until you close the compas_viewer window.

- 303_send_to_robot.py — only runs after you close the viewer, reads that same robot-ready file, connects live, fixes orientation, executes.

Missing:

- tile selector (result will be sent)

- record executed and write status for tile

to do:

- iput lins projected paths check for right origin right scaling(will be off because tile I send still has old origin) readability

- pull changes from lin and produce my own paths

- create own testing paths to receive and set up the mqtt

- clearly define what 302 process toolpath does

- clearly define what 303 process toolpath does

- compose 304 record execute and 305 tile execute

questions:

SB: what could be a good structure to set up 304 and 305

Joe – Gonzalo Mondays at 16:00 coding show and tell for compass – ask to get to know all the compass pros

SB: viewer probably in meters geo in mm so

Record executed – this needs to to be save hard!!! Store every x time as a file that contains all the information like time stamp type of spraying SB recommend .jason

Don’t forget loop that starts when there is new data received

Homework: object oriented programming look at classes, instances of that class, functions and more. Helps to keep things short and light.

Tuesday next check up and QA meeting

We can prepp the net manually with a clay mixture before we spray the substrate

Water could be with more distance potentially so it rather mists – next week distances tests for everything

Watering mode could just go left to right on the whole tile

Watering one json watering has a different robot function than spraying class has different things stored.

How many layers – one path 4 sprays? 1st spray 30 min than 3rounds back to back

What are the rules of spraying x3 set up some questions